# Multi-chanel Smart Meter (MSM) Firmware New Features Phase II

- Register a new device / Use an existing device

- Automated UID Fetching

- Data logging of Temperature

- Resolve false values of channel

- Implement Alerts/Alarms that has a possibility of it

  - Thresholds provided by the client:

  - Voltage unbalance (VUF) < 2 % continuous (IEEE 1159)

  - Current THD < 8 % (transformer ≤ K-factor 4), Voltage THD < 5 %

  - Sustained load < 80 % of nameplate kVA for > 30 min

  - PF > 0.95 lag (below this, schedule capacitor-bank check)

- Status indicators on the Device provided by the client are:

  - Modem

    - Modem not responding / Modem off

    - Modem booting

    - No SIM iv. SIM Locked

    - SIM Rejected

    - Connected to Network

  - Connection Type

    - 2G / 3G / 4G

  - Signal Strength

    - None / Poor / Fair / Good / Excellent

  - MQTT Connection

    - MQTT Failed – Timed out

    - MQTT Failed - Wrong credentials

    - MQTT Failed - Connection closed or unavailable

    - MQTT Success

    - Connection lost, Reconnecting to MQTT

  - MQTTs Connection

    - MQTTs Failed - Timed out

    - MQTTs Failed - Wrong credentials

    - MQTTs Failed - Connection closed or unavailable

    - MQTTs Success

    - Connection lost, Reconnecting to MQTTs

  - OTA

    - OTA In-Progress

    - OTA Failed

    - Application already on the selected version

- Implement login option when USB is inserted

- When disconnection or timeout occurs, data between cloud and device should not be missed

- Suggest & Implement OTA

- Load data to be fetched from database for new devices

- Remove GSM & LAN pages.

- Add MQTTs & OTA Screens for dynamic architecture

- Disable ports from editable

- Implement °C & °F switch of temperature. 
When I click on the temperature it should show temperature in Fahrenheit & on click again, in Celsius like a toggle functionality.